Smart Agriculture Monitoring System

API Reference Document

For: SE / CSE Mobile Application Team

From: ECE Team — Adama Science and Technology University (ASTU)  |  May 2026

# Quick Start

The ECE team has built and deployed a Node.js REST API that gives your mobile app access to all live sensor data and pump control. This document has everything you need to integrate.

Base URL (same WiFi network):

http://{ECE_TEAM_IP}:3000 Ask the ECE team for the current IP address when ready to integrate.

Test it immediately — open this in any browser:

http://{ECE_TEAM_IP}:3000/api/status

If you see JSON with "mqtt": "connected" — you are ready to go.

# 1. API Endpoints

| Method | Endpoint | What it returns | Use case |
| --- | --- | --- | --- |
| GET | /api/latest | Latest sensor reading + recommendation + timestamp | Refresh your app's main screen |
| GET | /api/history | Array of last 50 readings (newest last) | Plot charts in your app |
| POST | /api/pump | Success/failure message | ON/OFF button in your app |
| GET | /api/status | Server health + MQTT status + uptime | Check if ECE server is online |

# 2. Endpoint Details

## GET /api/latest

Returns the most recent sensor reading from the ESP32. Call this every 5–10 seconds to keep your app screen updated.

Request:

GET http://{ECE_TEAM_IP}:3000/api/latest

Response:

{ "success": true, "data": { "soil_moisture": 49, "temperature": 5.1, "humidity": 76, "light": 1001, "pump_status": 0, "recommendation": "Soil moisture acceptable (30-60%). Monitor closely. ¦ Low temperature (below 10°C). Risk of frost.", "timestamp": "2026-05-16T10:01:53.719Z" } }

Field reference:

| Field | Description |
| --- | --- |
| soil_moisture | Soil wetness percentage (0–100). Below 30 = dry, above 60 = good. |
| temperature | Air temperature in °C. One decimal place. |
| humidity | Relative humidity percentage (0–100). |
| light | Raw light intensity (0–4095). Higher = brighter. |
| pump_status | 0 = pump OFF, 1 = pump ON. |
| recommendation | Advisory text. May contain multiple messages separated by ¦ |
| timestamp | ISO 8601 UTC timestamp of when data was received by the server. |

## GET /api/history

Returns the last 50 sensor readings as a JSON array. Use this to draw charts or graphs in your app.

Request:

GET http://{ECE_TEAM_IP}:3000/api/history

Response:

{ "success": true, "count": 50, "data": [ { "soil_moisture": 45, "temperature": 5.1, "humidity": 74, "light": 998, "pump_status": 0, "timestamp": "2026-05-16T09:57:53.719Z" }, { ... more readings ... } ] }

- Readings are ordered oldest first, newest last.

- Each reading is published every 5 seconds in simulation (30 minutes on real hardware).

- The server keeps a rolling window — once 50 readings are stored, the oldest is dropped when a new one arrives.

## POST /api/pump

Sends a pump ON or OFF command. The server forwards it to the ESP32 via MQTT. The ESP32 relay activates within 1 second.

Request:

POST http://{ECE_TEAM_IP}:3000/api/pump Content-Type: application/json { "state": "ON" } -- or -- { "state": "OFF" }

Response (success):

{ "success": true, "message": "Pump turned ON" }

Response (error — wrong state value):

{ "success": false, "error": "state must be \"ON\" or \"OFF\"" }

- Important: state must be exactly "ON" or "OFF" (uppercase). Any other value returns a 400 error.

- If the ESP32 currently has soil moisture below 30%, the auto-irrigation logic may turn the pump back ON after the next sensor cycle (5 seconds). This is expected behavior.

## GET /api/status

Use this to check if the ECE server and MQTT connection are online before making other calls.

Request:

GET http://{ECE_TEAM_IP}:3000/api/status

Response:

{ "success": true, "server": "Smart Agriculture API", "mqtt": "connected", "uptime_sec": 295, "timestamp": "2026-05-16T08:43:59.935Z" }

| Field | Meaning |
| --- | --- |
| mqtt: "connected" | ECE server is receiving live data from the ESP32. Your app will get fresh data. |
| mqtt: "disconnected" | Broker connection dropped. Data may be stale. Contact ECE team. |
| uptime_sec | How long the server has been running in seconds. |

# 3. Code Examples

## JavaScript / React Native (fetch)

// Get latest sensor data const getLatest = async () => { const res = await fetch('http://{ECE_IP}:3000/api/latest'); const json = await res.json(); if (json.success) { console.log('Soil:', json.data.soil_moisture); console.log('Temp:', json.data.temperature); console.log('Advice:', json.data.recommendation); } }; // Turn pump ON const pumpOn = async () => { const res = await fetch('http://{ECE_IP}:3000/api/pump', { method: 'POST', headers: { 'Content-Type': 'application/json' }, body: JSON.stringify({ state: 'ON' }) }); const json = await res.json(); console.log(json.message); };

## Python (requests)

import requests BASE = 'http://{ECE_IP}:3000' # Get latest r = requests.get(f'{BASE}/api/latest') data = r.json()['data'] print(f"Soil: {data['soil_moisture']}%") print(f"Temp: {data['temperature']}°C") print(f"Advice: {data['recommendation']}") # Turn pump OFF r = requests.post(f'{BASE}/api/pump', json={'state': 'OFF'}) print(r.json()['message'])

## Dart / Flutter

import 'dart:convert'; import 'package:http/http.dart' as http; const baseUrl = 'http://{ECE_IP}:3000'; Future<Map> getLatest() async { final res = await http.get(Uri.parse('$baseUrl/api/latest')); return jsonDecode(res.body)['data']; } Future<void> setPump(String state) async { await http.post( Uri.parse('$baseUrl/api/pump'), headers: {'Content-Type': 'application/json'}, body: jsonEncode({'state': state}), ); }

# 4. Sensor Data Reference

| Sensor ID | Field Name | Range | Notes |
| --- | --- | --- | --- |
| ID 1 | soil_moisture | 0 – 100 (%) | Below 30 = dry, 30–60 = acceptable, above 60 = good |
| ID 2 | temperature | Any (°C) | Typical range 5–40°C in simulation |
| ID 3 | humidity | 0 – 100 (%) | Above 85% = fungal disease risk |
| ID 4 | light | 0 – 4095 (raw) | Raw ADC value from LDR sensor |
| ID 5 | pump_status | 0 or 1 | 0 = OFF, 1 = ON. Updated every publish cycle. |

# 5. MQTT Direct Connection (Optional)

If your app or backend prefers to connect directly to MQTT instead of calling the REST API, here are the broker details:

| Parameter | Value |
| --- | --- |
| Broker | broker.hivemq.com |
| Port | 1883 |
| Username | Team8_User |
| Password | SecurePass123 |
| Subscribe Topic | astu/agri/data (ESP32 publishes here every 5s) |
| Publish Topic | astu/agri/control (send "1" or "ON" to turn pump ON) |
| Message Format | JSON (see Section 4 for field names) |
| QoS | 1 recommended |

- You can subscribe to astu/agri/data and receive live data directly without going through the REST API.

- Note: The broker is public — do not transmit sensitive personal data over this connection.

# 6. Questions for the ECE Team

Before starting integration, please answer these questions so the ECE team can configure accordingly:

- Does your mobile app call our REST API directly, or do you have your own backend server?

- Are you using Firebase Realtime Database? If yes, the ECE team can push all sensor data directly to Firebase.

- Do you need the pump ON/OFF control from your app, or only data display?

- Are you on the same WiFi network as the ECE team, or do you need a public internet URL? (The ECE team can deploy to Railway/Render for free if needed.)

ECE Team Contact

| Name | Role |
| --- | --- |
| Beakal Bewnet Amdeberhan | IoT Hardware Lead |
| Meseret Kebede | Embedded Programmer |
| Abdisa Niguse | Power & Hardware Testing |
| Beakal Abreham | MQTT Communications |

Supervisor: Mr. Kedir Ibrahim |  Adama Science and Technology University (ASTU)

Date: May 19 2026